SPEAKER  RUANG

SPESIFIKASI TEKNIS PRODUK (Audio Paging System)

MASTER AMPLIFIER NUSTEC 40 CHANEL

MASTER AMPLIFIER NUSTEC 20 CHANEL

NusTec adalah perangkat

Audio Controller :

Adapun fungsi – fungsi yang dimaksud adalah :

- Pertama, sebagai Power Amplifier + Mixer atau pengatur masukan dan keluaran suara sekaligus sebagai penguatnya dengan kapasitas hingga 60 Speaker.

- Kedua, sebagai Audio Selector yang mengatur penyebaran keluaran suara yang dapat disesuaikan dengan bagian penerima suara yang dituju ( semua ruang atau acak ).

- Ketiga, sebagai Multimedia Input Terminal yang dapat memutarkan file-file suara dari media penyimpanan micro seperti : Flashdisk dan Memory Card.

Spesifikasi :

- Power Source AC 220-240 V 50/60 Hz

- Rated output 250 W

- Power consumption 124 W

- Frequency Response 50-18 , 000 Hz , + / - 3 dB

- Distortion Less than 0.3 % at 1 kHz , 1/3 Rated Power

- Input MIC 1 , MIC 2 , 3 MIC , MIC 4 -52 dBV ( 2.5 mV ) , 600 Ohm , socket Jack akay .

- AUX 1 , AUX 2 -10 dBV ( 316mV ) , 10 kOhm , not , RCA socket 2 Pin Jack

- Output Direct Line 4-16 Ohm , 42 Ohm ( 100 V )

- 1-36 -channel sound divider button ( For indoor ) , 3 swich hall ( Swich divider to the page ) and all of the buttons ( To turn all sound / speakers )

- S / N Ratio More than 65 dB ( MIC1- MIC3 ) , More than 75dB (AUX 1 - AUX2)

- Tone Control Lo : + / - 10 dB at 100 Hz , mid : +/- 10 db at 50 Hz , High : + / - 10 dB at 10 kHz

- Power indicator LED , LED Signal

- Panel Finish : Grey / Case : Steel Plate , Grey

- Dimensions 530 ( W ) x 180 ( H ) x 410 ( D ) mm

- Type : Condenser Microphone

- Directivity : Ultra - Cordioid

- Frequency Response : 80 - 12000Hz

- Sensitivity : 10mv/pa

- Output Impedance : 2KΩ

- Pick-up Distance : 5 -10 cm

- Power Source : DC 1.5 V x 2AA

Desk Microphone TARGA

- Rated Input Selection/Impedance : 6W,  (100V Line)

- Frequency Response : 100 ~ 12,000 Hz

- Sound Pressure Level (1W/1m) : 90dB

- Speaker Components : 16cm Dynamic Speaker (6 ")

- External Dimensions (WxHxD) : 290 x 214 x 150mm

- Weight product : 2.2kg

- Material : Enclosure: Wood, grille: Cloth

- Color : Enclosure: Light Grey, Grille : Light Grey

Speaker Monitor

Speaker Zone ( Indoor )

- Rated Input Selection/Impedance : 6W,  (100V Line)

- Frequency Response : 100 ~ 12,000 Hz

- Sound Pressure Level (1W/1m) : 90dB

- Speaker Components : 16cm Dynamic Speaker (6 ")

- External Dimensions (WxHxD) : 290 x 214 x 150mm

- Weight product : 2.2kg

- Material : Enclosure: Wood, grille: Cloth

- Color : Enclosure: Light Grey, Grille : Light Grey

Horn Speaker TOA ZH-615

( Out Door )

- Rated Input 25W (16)

- Frequency Response: 200~6.500 Hz

- Sound Preasure Level: 117dB (1W/1m, in anechoic chamber)

- Material: Horn: Aluminium, Color: Ash White

- Unit cover: ABS resin, Color: Dark Gray

- 30 W Mono Amplifier

- 220 V AC

- Input : 2 Aux, 2 Mic

- Solid Steel Case

- Dimensi : 17 cm X 5 cm X 14 cm

- Silver Only

Ampli pengajar ( teaching amplifier ) atu di pake juga untuk amplu upacara

- Sensitivitas		: -72dB

- Respon Frekuensi	: 100-10.000Hz

- Output		: 600 Ohm

- Tegangan Operasi	: 3V atau 1,5V

- Jarak Efektif		: 15-30m

- Isi 			: 1 pcs

Microphone Wireless Portable TARGA ( mic pengajar )

- Type : Dynamic Microphone

- Directivity : Undirectional

- Impedance : 600 Ohms

- Sensitivity : - 76 dB

Microphone Kabel BS 828 ( mik upacara )

Spesifikasi  Stand Mic :

- Kaki Bulat

- Bahan Besi

- Putaran 360 derajat

- Tinggi min = 100cm

- Tinggi Maks =.160cm

- Finishing      =Warna Hitam

Stand Mic Black Spider

( MS – 84 )

Speaker CEILLING ZS-646R

- Flush Ceiling Mount

- Respon Frekuensi	 : 100-12.000Hz

- Input Rata-Rata	 : 6W

- Rated Input Selection / Impedansi : 6W,3W,1.5W,0.8W (100VLine)

- Dimensi Eksternal 	 : 191 x 74mm

- Berat		 : 610 g

- Speaker Mounting Hole: 150 mm

- Hasil akhir 		 : Panel; ABS Resin, Off White

LOGO / MERK DAGANG